FUDGE :     You may or may not choose to run this program. I use this to

identify potential tracking problems with portion of a pass

not being trackable. Before running it you need two items of

information - the time ( in seconds) that it takes your mount

to go through 360 degrees in azimuth and 90 degrees in elevation.

- for example my particular values are 40 and 105 seconds.

The program will then produce an OUT file for each satellite.

The data shown is quite simple and is as follows:

Az(N+1)   Az(N)  ChangeAz  TimeAz(sec)  El(N+1)  El(N)  ChangeEl TimeEl(sec)

AZ(N+1) is the azimuth reading after AZ(N) so ChangeAz will tell you how much

the azimuth changes between successive predictions. Timeaz(sec) will tell you

if your mount can move that amount. If the value is greater than 1.00 then

the satellite is moving too fast to keep up with.

Similarly El(N+1) is the next elevation reading after El(N), ChangeEL will

indicate how much the elevation has changed between successive predictions

and TimeEl(sec) will indicate if you can keep up - I have yet to see a

prediction where the elevation changes too fast, so the main "crash" item

is the change in azimuth.

If the TimeAz(sec) is greater than 1.00 I suggest you only keep part of the

prediction that is less than 1.00. This means that you will have two parts

(usually) to the prediction - the first where the satellite is ascending to

culmination and the second with the satellite descending from culmination.

You have to decide which you want - I will modify the program to split the

prediction and give you two predictions for the object - one ascending, the

other descending. Meanwhile I usually go for the part of the pass where the

phase angle is greatest ( since the satellite may then be brighter)-however

as winter approaches the satellite will start moving into earths shadow so

in some cases its best to choose the ascending part of the pass.

Greg Roberts

16 April 2000

PS  Since this program was written CoSaTrak has FUDGE built into it so it is

no longer necessary to split a prediction pass. CoSaTrak does it much

more elegantly.